⚙  Engineering Calculator Toolkit

Python Engineering Utility Tool  ·  v1.0

A modular Python toolkit for structural and mechanical engineering calculations, featuring a dark-themed interactive GUI built with Tkinter. Zero external dependencies for the core and UI — just Python 3.8+.

## Project Structure

engineering_toolkit/ │ ├── main.py ← Entry point (GUI / CLI / Tests) ├── requirements.txt │ ├── core/ ← Pure logic, zero UI dependency │ ├── __init__.py │ ├── beam.py ← Beam deflection calculator │ ├── stress.py ← Stress, strain, Von Mises, FoS │ ├── inertia.py ← Section properties (Ixx, Iyy, J, Sx, Sy) │ └── units.py ← 10-category, 80+ unit converter │ ├── ui/ │ └── app.py ← Tkinter GUI — 4 tabbed calculators │ └── tests/ └── test_toolkit.py ← 25 unit tests

## Quick Start

macOS / Linux: Use python3 and pip3

### Launch the GUI

cd engineering_toolkit python3 main.py

### CLI interactive mode  (no GUI needed)

python3 main.py --cli

### Run tests

pip3 install pytest python3 -m pytest tests/ -v # or via the launcher python3 main.py --test

## Modules

### 1 · Beam Deflection  —  core/beam.py

Calculates maximum deflection, bending moment, and shear force for three beam configurations under two load types.

Beam types supported:

- Simply Supported

- Cantilever

- Fixed-Fixed

Load types:

- Point Load (centre or off-centre)

- UDL — Uniformly Distributed Load

Outputs:

- Max deflection in mm with formula used

- Max bending moment (N·m) and shear force (N)

- L/360 serviceability check — warns if exceeded

### 2 · Stress & Factor of Safety  —  core/stress.py

Calculates stress, strain, and factor of safety for multiple loading modes.

Stress types:

- Normal (axial)  σ = F/A

- Bending (flexural)  σ = My/I

- Shear  τ = V/A  (Tresca criterion)

- Torsional  τ = Tr/J

- Von Mises equivalent stress for combined loading

- Thermal stress  σ = E·α·ΔT

Safety check: FoS ≥ 1.5 → SAFE, below 1.5 → UNSAFE (industry standard minimum).

### 3 · Moment of Inertia  —  core/inertia.py

Computes full section properties using exact formulas and the parallel axis theorem for composite sections.

Sections supported:

- Rectangle

- Solid Circle

- Hollow Circle (tube)

- I-Section (H-Beam)  — parallel axis theorem applied

- T-Section  — centroid computed first (asymmetric)

- Triangle

Outputs:

- Area (m²), Centroid location

- Ixx, Iyy (second moment of area), J (polar)

- Section modulus Sx, Sy

- Radius of gyration rx, ry

### 4 · Unit Converter  —  core/units.py

Converts between 80+ engineering units across 10 categories. Temperature uses offset-based conversion; all other categories use SI multipliers.

| Category | Base Unit | Example Units | Count |
| --- | --- | --- | --- |
| Force | N | kN, MN, lbf, kip, kgf | 8 |
| Pressure | Pa | MPa, GPa, bar, psi, atm | 11 |
| Length | m | mm, cm, km, in, ft, mi | 10 |
| Mass | kg | g, tonne, lb, slug | 9 |
| Temperature | K | °C, °F, K, °R | 4 |
| Energy | J | kJ, kWh, cal, BTU | 10 |
| Power | W | kW, MW, hp, BTU/hr | 7 |
| Angle | rad | deg, grad, arcmin | 6 |
| Area | m² | mm², cm², in², ft², ha | 9 |
| Torque | N·m | kN·m, lbf·ft, kgf·m | 7 |

## Engineering Concepts Applied

| Concept | How It's Implemented |
| --- | --- |
| Numerical Computation | Deflection integrals (PL³/48EI), parallel axis theorem (I = I₀ + Ad²), Von Mises tensor |
| Formula Modelling | Python Enum dispatch selects the correct textbook formula at runtime based on beam/load type |
| Scientific Scripting | Dataclasses for structured results, type hints throughout, SI unit system as the canonical base |
| Engineering Logic Translation | L/360 serviceability limit, Tresca shear criterion (τ_y = σ_y/2), FoS < 1.5 = unsafe flag |

## Key Design Decisions

### Core / UI separation

The core/ package has zero Tkinter dependency. Every function takes plain numbers and returns a dataclass. This means the GUI in ui/app.py can be replaced with a Flask web server or FastAPI endpoint without touching a single formula.

### Enum-driven formula dispatch

BeamType and LoadType are Python Enums, not strings. This makes it impossible to pass invalid types and gives VS Code full autocomplete on every call.

### Dataclass return types

All calculators return structured dataclasses (BeamResult, StressResult, SectionProperties) so the UI can access .factor_of_safety or .warning individually — no string parsing needed.

### T-section centroid first

Unlike symmetric sections, the T-section centroid is not at mid-height. The code computes the centroid from the bottom using the composite area method before applying the parallel axis theorem — a step students commonly miss.

## Dependencies

✓ Core + GUI: Python 3.8+ standard library only (math, tkinter, dataclasses, enum)

+ Tests only: pip3 install pytest

## Extending the Toolkit

To add a new cross-section type:

- Add a function in core/inertia.py

- Add an entry to the SectionType enum

- Add to the dispatch dict in calculate_section()

- Add UI fields in InertiaTab._update_fields() in ui/app.py

To add a new unit category:

- Add a new dict entry to CONVERSIONS in core/units.py

- The converter and UI pick it up automatically

Engineering Calculator Toolkit  ·  Python 3.8+  ·  MIT License